SWMM5 Lego Builder

Build - Validate - Simulate - Export

Browser-Based Stormwater Management Modeling Tool

https://swmm-lego-model.replit.app/

Documentation v1.0 - March 2026

# Table of Contents

Right-click the TOC and select "Update Field" to refresh page numbers

Product Overview	3

Key Features	4

Surface Types	4

Node Elements	5

Link Elements	5

Workflow Guide	6

Simulation Engines	7

Design Storms	8

Keyboard Shortcuts	9

# Product Overview

SWMM5 Lego Builder is a browser-based stormwater management modeling tool that allows engineers, planners, and students to build, validate, simulate, and export stormwater networks directly in their web browser. Inspired by the intuitive nature of LEGO building blocks, the application provides a visual, drag-and-drop interface for creating complex hydraulic models without requiring installation of traditional desktop software.

The application combines two powerful simulation engines: a simplified JavaScript-based Quick Sim engine for rapid prototyping and visualization, and the full EPA SWMM5 v5.2 solver compiled to WebAssembly for comprehensive hydraulic analysis. This dual-engine approach enables users to iterate quickly during the design phase while still having access to industry-standard simulation capabilities.

Whether you are designing green infrastructure, analyzing urban drainage systems, or teaching hydrology concepts, SWMM5 Lego Builder provides an accessible yet powerful platform for stormwater modeling.

# Key Features

## Surface Types

The application provides nine distinct surface types that can be painted onto the modeling grid. Each surface type has unique hydrological properties that affect runoff generation:

| Element | Description |
| --- | --- |
| Grass | Natural pervious surface with standard infiltration |
| Roof | Impervious building roof surface |
| Road | Paved roadway with high imperviousness |
| Driveway | Residential paved surface |
| Sidewalk | Pedestrian walkway surface |
| LID Pond | Low Impact Development bioretention pond |
| Perm Pave | Permeable pavement system |
| Green Roof | Vegetated rooftop system |
| Rain Barrel | Rainwater harvesting system |
| Swale | Grassed drainage channel |

Table 1: Available Surface Types

## Node Elements

Five types of hydraulic nodes can be placed on the grid to represent collection and control points in the drainage system:

| Element | Description |
| --- | --- |
| Manhole | Standard junction for pipe connections |
| Inlet | Stormwater inlet for surface collection |
| Outfall | System discharge point |
| Storage | Detention or retention facility |
| Divider | Flow splitting junction |

Table 2: Available Node Types

## Link Elements

Five types of hydraulic links connect nodes and convey flow through the system:

| Element | Description |
| --- | --- |
| Pipe | Closed conduit for underground flow |
| Channel | Open channel for surface flow |
| Pump | Mechanical flow lift station |
| Orifice | Controlled outlet structure |
| Weir | Overflow or spillway structure |

Table 3: Available Link Types

# Workflow Guide

The SWMM5 Lego Builder follows a clear eight-step workflow for creating and analyzing stormwater models:

- Paint Surfaces: Select surface types and paint them onto the grid to represent land use

- Place Nodes: Add manholes, inlets, outfalls, storage, and dividers to the model

- Connect Links: Draw pipes, channels, pumps, orifices, and weirs to connect nodes

- Edit Properties: Right-click on any element to modify its hydraulic parameters

- Validate Model: Run the validation tool to check for errors and inconsistencies

- Fix Issues: Use the auto-fix feature or manually correct any identified problems

- Run Simulation: Execute either Quick Sim or EPA SWMM5 simulation

- Export Results: Save the model or export to .inp format for use in EPA SWMM5

# Simulation Engines

## Quick Sim (JavaScript)

The Quick Sim engine provides rapid simulation using simplified JavaScript-based hydrology. It implements the SCS Curve Number method for runoff generation and Manning's equation for flow routing. With a 15-second time step and real-time animation, Quick Sim is ideal for educational purposes and rapid design iteration.

## EPA SWMM5 v5.2 (WASM)

The full EPA SWMM5 solver is compiled to WebAssembly (WASM) and runs entirely in the browser. This engine provides complete Dynamic Wave routing capabilities and generates standard SWMM5 report (.rpt) output files. Users can export models and results for further analysis in desktop SWMM5 applications.

Table 4: Engine Comparison

| Feature | Quick Sim | EPA SWMM5 |
| --- | --- | --- |
| Engine | JavaScript | WebAssembly (WASM) |
| Hydrology | SCS-CN | SCS-CN + Green-Ampt |
| Routing | Manning's Equation | Dynamic Wave |
| Time Step | 15 seconds | Variable |
| Animation | Yes | No |
| Output | Visual + CSV | RPT + CSV |
| Best For | Rapid prototyping | Detailed analysis |

# Design Storms

The application includes 49 pre-configured design storms from various international standards and regional specifications. Storms are organized by geographic region:

### United States Standards

SCS Type II (standard for ~95% of continental US), SCS Type III, SCS Type I, SCS Type IA, Balanced (Alt Block), NOAA Atlas 14, Huff 1st-4th Quartile, USACE Standard Project, PMP (HMR 51/52), FDOT Zones 1-5 (Florida), TxDOT Empirical, UDFCD Denver

### European Standards

Triangular (UK FSR), Trapezoidal, FSR Profile (75%), FEH Temporal, Chicago Storm, Desbordes, Desbordes Double Triangular, DWA-A 531 (Germany), Euler Type I & II (Germany), Dutch STOWA, Italian Mediterranean

### Asian & Pacific Standards

Japan JMA, AMeDAS Convective, Baiu Frontal, Japan Typhoon, China Design Storm, China GB 50014-2021, Pearl River Delta, India IMD Monsoon, India Coastal Cyclonic, Korea KMA Standard, Singapore PUB, Australian ARR

### Other International

South African Huff, Canadian CDA/MTO, Block (Uniform), Double Peak, Yen & Chow Triangular, Custom (Uniform)

# Keyboard Shortcuts

The following keyboard shortcuts enhance productivity when working with the model:

| Shortcut | Action |
| --- | --- |
| Ctrl+Z | Undo last action |
| Space | Toggle Paint/Erase mode |
| Del | Activate erase mode |
| R | Run simulation |
| Esc | Close panels |
| 1-9 | Select surface type |
| Shift+1-5 | Select node type |
| Q/W/E | Select Pipe/Channel/Pump |

Table 5: Keyboard Shortcuts

# Technical Specifications

## Grid Sizes

Models can be built on grids ranging from 20x20 to 50x50 cells. The grid size determines the spatial resolution of the model and should be selected based on the scale of the study area and required detail level.

## Subcatchment Parameters

Each surface type is automatically assigned SCS Curve Number hydrology parameters including:

| Element | Description |
| --- | --- |
| Curve Number | Runoff curve number (0-100) |
| % Impervious | Percentage of impervious area |
| n-Imperv | Manning's n for impervious surfaces |
| n-Perv | Manning's n for pervious surfaces |
| Ds-Imperv | Depression storage on impervious areas |
| Ds-Perv | Depression storage on pervious areas |
| S (retention) | Potential maximum retention |
| Ia (init abs) | Initial abstraction |
| Runoff Coeff | Runoff coefficient |

Table 6: Subcatchment Parameters

SWMM5 Lego Builder

Build stormwater networks like LEGO

https://swmm-lego-model.replit.app/

SWMM5.org  |  EPA SWMM5 WASM + Quick Sim

© 2026 - The Dickinson Canon